Caster Angle=+4.00∘

KPI=1.99∘≈2.00∘
Static Camber Angle=-1.99∘≈-2.00∘

| Point Variable | X (mm)(Front/Rear) | Y (mm)(Center/Outward) | Z (mm)(Height above Ground) | Physical Component Location |
| --- | --- | --- | --- | --- |
| Wheel_Center | 0.0 | 600.0 | 228.0 | Axle hub center (Tire radius = 228 mm) |
| LCA_outboard | 0.0 | 560.0 | 115.0 | Lower ball joint inside wheel rim |
| UCA_outboard | -13.3 | 553.4 | 305.0 | Upper ball joint inside wheel rim |
| LCA_inboard_F | -180.0 | 220.0 | 110.0 | Front mounting point of Lower Control Arm |
| LCA_inboard_R | 180.0 | 220.0 | 110.0 | Rear mounting point of Lower Control Arm |
| UCA_inboard_F | -150.0 | 270.0 | 270.0 | Front mounting point of Upper Control Arm |
| UCA_inboard_R | 150.0 | 270.0 | 270.0 | Rear mounting point of Upper Control Arm |
| Pushrod_LCA | 0.0 | 450.0 | 113.5 | Pushrod mounting point on Lower Wishbone |
| Pushrod_Rocker | -20.0 | 240.0 | 360.0 | Pushrod attachment to Rocker bellcrank |
| Rocker_Pivot | -20.0 | 210.0 | 350.0 | Rocker pivot axle on upper chassis |
| Damper_Rocker | -20.0 | 175.0 | 385.0 | Top shock eyelet on Rocker bellcrank |
| Damper_Chassis | -20.0 | 110.0 | 240.0 | Bottom shock eyelet on chassis frame |
| Tierod_inner | -80 | 210 | 180 | Inner Tie rod attached to steering |
| Tierod_outer | -80 | 550 | 180 | Outer tie rod attached to wheel |